# NASA Space Mission AI Project Plan - Conceptual Design Track

## Project Overview

Student Name: Andrew Kuruvilla
Project Track: Conceptual Design Track
Project Name: Andrew_Kurvilla_Conceptual_Design_Track
GitHub Repository: https://github.com/drewvilla1/Andrew_Kurvilla_Conceptual_Design_Track

Project Description: This project proposes a detailed conceptual design for an AI system to optimize autonomous navigation for a NASA space mission. The AI solution will focus on real-time path planning and obstacle avoidance for a Mars rover, leveraging machine learning and sensor data fusion to enhance mission efficiency and safety.

## Project Proposal

### Problem Statement

The exploration of Mars requires rovers to navigate complex terrains with minimal human intervention due to communication delays. Current navigation systems rely on pre-programmed routes and limited real-time adaptability, which can lead to inefficiencies or mission risks when encountering unexpected obstacles (e.g., rocks, craters, or sand traps).

### Proposed Solution

Develop an AI-based autonomous navigation system for a Mars rover that:

- Uses a combination of computer vision, reinforcement learning, and sensor fusion to dynamically plan paths.

- Processes data from LiDAR, stereo cameras, and inertial measurement units (IMUs) to detect and classify obstacles.

- Adapts to changing environmental conditions (e.g., dust storms, lighting changes) in real-time.

- Optimizes for energy efficiency and mission objectives (e.g., reaching scientific targets).

### Objectives

- Design an AI model architecture for real-time path planning.

- Create a sensor fusion framework to integrate multiple data sources.

- Develop a testing plan to validate the system under simulated Mars conditions.

- Ensure the system is robust, scalable, and aligns with NASA’s mission requirements.

### Scope

- Focus on navigation and obstacle avoidance for a Mars rover.

- Exclude onboard scientific analysis or communication systems.

- Assume a simulated environment for testing and validation.

## Detailed Solution Plan

### System Architecture

The proposed AI system consists of the following components:

- Data Acquisition Module:

  - Inputs: LiDAR point clouds, stereo camera images, IMU data.

  - Function: Collects and preprocesses raw sensor data for analysis.

  - Technology: Python-based data pipelines, OpenCV for image processing, PCL (Point Cloud Library) for LiDAR.

- Sensor Fusion Module:

  - Approach: Use a Kalman filter to integrate LiDAR, camera, and IMU data for accurate environmental mapping.

  - Output: A unified 3D map of the rover’s surroundings.

  - Rationale: Combining multiple sensor inputs improves robustness against noise and environmental variability.

- Obstacle Detection and Classification:

  - Model: Convolutional Neural Network (CNN) trained on labeled Mars terrain data.

  - Function: Identifies obstacles (e.g., rocks, slopes) and classifies them by risk level.

  - Training Data: Simulated Mars terrain datasets (e.g., NASA’s Mars Yard) and synthetic data augmentation.

- Path Planning Module:

  - Algorithm: Reinforcement Learning (RL) with a Deep Q-Network (DQN) to optimize paths.

  - Inputs: 3D map, obstacle classifications, mission objectives.

  - Outputs: Optimal path avoiding obstacles while minimizing energy use.

  - Rationale: RL allows the rover to learn from experience and adapt to new terrains.

- Control Interface:

  - Function: Translates planned paths into rover motor commands.

  - Technology: ROS (Robot Operating System) for communication between AI and rover hardware.

### Workflow

- Sensors collect raw data (LiDAR, cameras, IMU).

- Data is preprocessed and fused into a 3D environmental map.

- CNN identifies and classifies obstacles in the map.

- RL-based path planner generates an optimal route.

- Control interface sends commands to the rover’s motors.

- System continuously updates based on new sensor data.

### Assumptions

- Rover hardware supports real-time data processing (e.g., onboard GPU).

- Simulated Mars environment is available for testing.

- NASA provides access to relevant terrain datasets for training.

### Risk Mitigation

- Risk: Sensor noise or failure.

  - Mitigation: Implement redundancy in sensor fusion and fallback to conservative navigation if data quality drops.

- Risk: Model overfitting to simulated data.

  - Mitigation: Use data augmentation and domain randomization during training.

- Risk: Computational limitations.

  - Mitigation: Optimize algorithms for efficiency and prioritize critical tasks.

## Testing Plan

### Testing Objectives

- Validate the accuracy of obstacle detection and classification.

- Ensure path planning adapts to dynamic environments.

- Confirm system robustness under simulated Mars conditions.

- Verify energy efficiency and mission objective alignment.

### Test Scenarios

- Static Obstacle Avoidance:

  - Environment: Simulated Mars terrain with rocks and craters.

  - Objective: Rover navigates to a target without collisions.

  - Metric: Success rate (target reached without collision).

- Dynamic Environment:

  - Environment: Terrain with changing conditions (e.g., dust storm reducing visibility).

  - Objective: Rover adjusts path in real-time.

  - Metric: Path adaptation time and collision avoidance.

- Edge Cases:

  - Environment: Extreme slopes, narrow passages, or sensor noise.

  - Objective: Rover avoids mission failure (e.g., getting stuck).

  - Metric: Failure rate and recovery time.

- Energy Efficiency:

  - Environment: Long-distance navigation with multiple obstacles.

  - Objective: Minimize energy consumption while reaching target.

  - Metric: Energy usage compared to baseline (e.g., shortest path).

### Testing Tools

- Simulation Platform: Gazebo with ROS for Mars terrain simulation.

- Dataset: NASA’s Mars Yard dataset and synthetic terrain data.

- Metrics Tracking: Custom Python scripts to log success rates, energy usage, and adaptation times.

### Validation Criteria

- Obstacle detection accuracy: >90% precision and recall.

- Path planning success rate: >95% in static environments, >85% in dynamic environments.

- Energy efficiency: At least 20% improvement over shortest-path baseline.

- Robustness: No mission failures in >90% of edge case tests.